++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

+  MTPCKG  MTPCKG  MTPCKG  MTPCKG  MTPCKG  MTPCKG  MTPCKG  MTPCKG  MTPCKG  +

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

ROBERT S. DEVERILL  12 JULY 1983

USAGE:  $ LINK ...,PUB:CPROCS/OPT

THE SUBROUTINE PACKAGE, MTPCKG, USES THE BASIC PROCEDURES OF THE

VAX/VMS OPERATING SYSTEM, VIA FORTRAN IV-PLUS, TO IMPLEMENT A SET OF MAG-

NETIC TAPE I/O PRIMITIVES APPLICABLE TO TE-16 9-TRACK TAPE DRIVES.  EACH

OF THESE PRIMITIVES RETURNS A RESULT CODE, A SMALL NON-NEGATIVE INTEGER,

TO INDICATE THE MODE OF COMPLETION OF THE REQUESTED OPERATION.  A SUCCESS-

FUL COMPLETION IS SIGNALLED BY RESULT = 0;  THE VARIOUS UNUSUAL OR

UNSUCCESSFUL COMPLETIONS ARE SIGNALLED BY POSITIVE VALUES OF THE RESULT CODE.

DEFINITIONS OF THE POSSIBLE NON-ZERO RESULT CODES FOR THE PRIMITIVES OF

MTPCKG CAN BE OBTAINED FROM THE MODULE 'SRC:MTERCS.INS', WHICH MAY BE COPIED

OR INCLUDED IN THE FORTRAN SOURCE CODE OF THE MTPCKG USER.  HERE ARE THE

RESULT CODE DEFINITIONS THAT WOULD BE SO OBTAINED:__

C----------  ERROR CODES FOR 'MTOPEN'  ----------

PARAMETER MTRNDV = 1             ! NO AVAILABLE TAPE DRIVE

PARAMETER MTRNCH = 2             ! NO AVAILABLE I/O CHANNEL

PARAMETER MTRNEF = 3             ! NO AVAILABLE EVENT FLAG

C----------  ERROR CODES FOR 'MTCLOS'  ----------

C----------  ERROR CODES FOR 'MTNOOP'  ----------

C                        (NONE)

C----------  ERROR CODES FOR 'MTSERM'  ----------

PARAMETER MTRERU = 1             ! ERROR MODE UNCHANGED

C----------  ERROR CODES FOR 'MTNAME'  ----------

PARAMETER MTRTRN = 1             ! TRUNCATED NAME

C----------  ERROR CODES FOR 'MTREAD'  ----------

C----------  ERROR CODES FOR 'MTRDBK'  ----------

C----------  ERROR CODES FOR 'MTSKRE'  ----------

PARAMETER MTRERR = 1             ! I/O ERROR

PARAMETER MTROFL = 2             ! TAPE OFFLINE

PARAMETER MTREOF = 3             ! END OF FILE

PARAMETER MTREOT = 4             ! END OF TAPE OR

! BEGINNING OF TAPE

PARAMETER MTRNOR = 5             ! NO RECORD

C----------  ERROR CODES FOR 'MTWRIT'  ----------

C----------  ERROR CODES FOR 'MTWEOF'  ----------

C     PARAMETER MTRERR = 1             ! I/O ERROR

C     PARAMETER MTROFL = 2             ! TAPE OFFLINE

PARAMETER MTRWLK = 3             ! WRITE LOCK

C     PARAMETER MTREOT = 4             ! END OF TAPE

C----------  ERROR CODES FOR 'MTSKFI'  ----------

C     PARAMETER MTRERR = 1             ! I/O ERROR

C     PARAMETER MTROFL = 2             ! TAPE OFFLINE

C                        3             ! (UNUSED)

C     PARAMETER MTREOT = 4             ! END OF TAPE OR

C                                      ! BEGINNING OF TAPE

C     PARAMETER MTRNOR = 5             ! NO RECORD

C----------  ERROR CODES FOR 'MTREWI'  ----------

C----------  ERROR CODES FOR 'MTUNLD'  ----------

C     PARAMETER MTRERR = 1             ! I/O ERROR

C     PARAMETER MTROFL = 2             ! TAPE OFFLINE

C----------  ERROR CODES FOR 'MTPREP'  ----------

C     PARAMETER MTRERR = 1             ! I/O ERROR

C     PARAMETER MTROFL = 2             ! TAPE OFFLINE

PARAMETER MTRBDN = 3             ! ILLEGAL DENSITY

C----------  ERROR CODES FOR 'MTSENS'  ----------

C     PARAMETER MTRERR = 1             ! I/O ERROR

C     PARAMETER MTROFL = 2             ! TAPE OFFLINE

PARAMETER MTRVIN = 3             ! VOLUME INVALID

C----------  ERROR CODES FOR 'MTSETW'  ----------

PARAMETER MTRWMU = 1             ! WAIT MODE UNCHANGED

C----------  ERROR CODES FOR 'MTWAIT'  ----------

C----------  ERROR CODES FOR 'MTCHCK'  ----------

C     AS FOR THE CORRESPONDING FUNCTION, ABOVE.

MTPCKG HAS 18 ENTRY POINTS:

CALL MTOPEN(ID, RESULT) -- TO ALLOCATE AND CONNECT A TAPE DRIVE TO THE

=======================    CALLING PROCESS.

ID     - INTEGER:  (OUTPUT) RECEIVES AN INTEGER VALUE FROM MTOPEN

WHICH IS TO BE USED TO IDENTIFY THE PARTICULAR TAPE DRIVE

WHICH HAS BEEN ALLOCATED TO YOUR USE BY THIS CALL.  THE

VALUE OF ID IS USED AS AN INPUT PARAMETER TO EVERY SUBSE-

QUENT CALL UPON THE I/O PRIMITIVES OF MTPCKG.  BY SEVERAL

CALLS OF MTOPEN YOU MAY ALLOCATE UP TO 8 TAPE DRIVES, OR

UP TO AS MANY TAPE DRIVES AS ARE AVAILABLE IF THERE ARE LESS

THAN 8 OF THEM.

RESULT - INTEGER:  (OUTPUT) RECEIVES AN INDICATION OF SUCCESS OR

FAILURE.  THE MODES OF FAILURE CAN BE AS FOLLOWS:

RESULT=1 - ALL OF THE TAPE DRIVES HAVE BEEN ALLOCATED

TO YOU OR TO OTHERS, SO THAT THERE IS NONE

TO FULFILL THIS REQUEST.

RESULT=2 - THE OPERATING SYSTEM HAS RUN OUT OF I/O CHAN-

NELS AND THEREFORE CANNOT CONNECT YOU WITH

THE ALLOCATED TAPE DRIVE.

RESULT=3 - THE OPERATING SYSTEM RUN OUT OF EVENT FLAGS

AND THEREFORE LACKS THE MEANS OF SYNCHRONIZ-

ING I/O OPERATIONS ON THE ALLOCATED CHANNEL.

IF RESULT IS .NE. 0, THEN THE ID VALUE IS INVALID FOR ANY

SUBSEQUENT CALLS OF MTPCKG.

CALL MTCLOS(ID, RESULT) -- TO DEALLOCATE THE TAPE DRIVE ASSOCIATED WITH

=======================    THIS VALUE OF ID.  IF THERE IS A TAPE REEL

CURRENTLY MOUNTED ON THE DRIVE IT IS NOT REWOUND OR UNLOADED.

THE ID VALUE BECOMES INVALID FOR FURTHER USE IN MTPCKG.

RESULT - INTEGER:  (OUTPUT) IS ALWAYS ZERO.

CALL MTNAME(ID, NAME, LENGTH, RESULT) -- TO OBTAIN THE PHYSICAL NAME OF

=====================================    THE ACTUAL TAPE DRIVE THAT IS

ALLOCATED TO THIS VALUE OF ID.

NAME   - CHARACTER:  (OUTPUT) RECEIVES THE NAME STRING OF THE ALLOC-

ATED TAPE DRIVE.  IF NAME IS NOT LONG ENOUGH TO HOLD THE

COMPLETE NAME STRING, THEN THE NAME STRING IS TRUNCATED AT

THE RIGHT SO THAT WHAT IS LEFT OF IT WILL FIT:  NAME SHOULD

HAVE LENGTH AT LEAST 6 TO AVOID THIS.

LENGTH - INTEGER:  (OUTPUT) RECEIVES THE ACTUAL LENGTH OF THE NAME

STRING.

RESULT - INTEGER:  (OUTPUT) CAN RECEIVE A NON-ZERO WARNING CODE:

RESULT=1 - NAME IS NOT LONG ENOUGH TO HOLD ALL OF THE

NAME STRING.

CALL MTSENS(ID, STATUS, RESULT) -- TO OBTAIN INFORMATION ABOUT THE CURRENT

===============================    STATUS OF THE TAPE DRIVE ASSOCIATED WITH

THIS VALUE OF ID.

STATUS - INTEGER:  (OUTPUT) RECEIVES 32 BITS OF STATUS INFORMATION:

00000000000ABCDE000FFFFFGGGGH000

WHICH HAVE THE FOLLOWING SIGNIFICANCE:

A    =1     - TAPE POSITION LOST

=0     - TAPE POSITION OK

B    =1     - TAPE IS WRITE LOCKED

=0     - TAPE IS NOT WRITE LOCKED

C    =1     - END-OF-TAPE MARKER ENCOUNTERED

=0     - NOT END-OF-TAPE

D    =1     - END-OF-FILE ENCOUNTERED

=0     - NOT END-OF-FILE

E    =1     - BEGINNING OF TAPE

=0     - NOT BEGINNING OF TAPE

FFFFF=00011 - DENSITY IS 800 BPI

=00100 - DENSITY IS 1600 BPI

=00101 - DENSITY IS 6250 BPI

GGGG =1100  - TAPE FORMAT IS NORMAL 11

0000  - TAPE FORMAT IS DEFAULT (NORMAL 11)

H    =1     - EVEN PARITY

=0     - ODD PARITY.

RESULT - INTEGER:  (OUTPUT) CAN RECEIVE A NON-ZERO RETURN CODE:

RESULT=1 - AN I/O ERROR OCCURRED WHILE OBTAINING THE

STATUS

RESULT=2 - THE TAPE DRIVE IS OFFLINE

RESULT=3 - THE TAPE DRIVE IS ONLINE BUT HAS BEEN OFFLINE

SOMETIME BETWEEN NOW AND WHEN YOU LAST CALL-

ED MTPCKG, I.E., THE TAPE REEL HAS BEEN

MANUALLY REWOUND OR UNLOADED OR SWITCHED.

CALL MTPREP(ID, DENSIT, RESULT) -- TO SET THE TAPE DENSITY FOR WRITING.

===============================    THIS SHOULD BE DONE WHEN THE WRITING

IS TO COMMENCE AT THE BEGINNING OF THE TAPE.  THE DEFAULT

DENSITY IS 800 BPI.  IF THE TAPE HAS BEEN READ OR SPACED,

THEN THE DENSITY WILL HAVE BEEN SET TO WHATEVER VALUE PRE-

VAILED ON THE TAPE ITSELF.  MTPREP REWINDS THE TAPE SO THAT

IT IS IMPOSSIBLE TO WRITE A TAPE WITH RECORDS OF VARYING DEN-

SITY.

DENSIT - INTEGER: THE VALUE, 800, 1600 OR 6250, FOR THE RECORDING

DENSITY.

RESULT - INTEGER:  (OUTPUT) CAN RECEIVE NON-ZERO VALUES:

RESULT=1 - AN I/O ERROR OCCURRED

RESULT=2 - THE TAPE DRIVE IS OFFLINE

RESULT=3 - THE DENSITY VALUE WAS OTHER THAN 800, 1600

OR 6250.

CALL MTSERM(ID, INHERC, RESULT) -- TO SET THE I/O ERROR RECOVERY MODE.

===============================

INHERC - LOGICAL:  .TRUE. TO INHIBIT ERROR RECOVERY PROCEDURES ON

I/O ERRORS;  .FALSE. FOR NORMAL ERROR RECOVERY ATTEMPTS.

THE ERROR RECOVERY MODE SET BY THIS FUNCTION WILL REMAIN

IN EFFECT UNTIL CHANGED BY ANOTHER CALL OF MTSERM.  THE

DEFAULT ERROR RECOVERY MODE IS RETRY (INHERC = .FALSE.).

RESULT - INTEGER:  (OUTPUT) POSSIBLE NON-ZERO VALUES:

RESULT=1 - THE ERROR RECOVERY MODE IS UNCHANGED, I.E.,

THE ERROR RECOVERY MODE SPECIFIED BY THE

PARAMETER, INHERC, WAS ALREADY IN EFFECT.

CALL MTREAD(ID, BUFLEN, BUFFER, ACTLEN, RESULT) -- TO READ THE NEXT RECORD

===============================================    IN THE FORWARD DIREC-

TION FROM THE CURRENTLY LOADED TAPE REEL INTO THE BUFFER.

BUFLEN - INTEGER:  THE LENGTH, IN BYTES, OF THE BUFFER AREA INTO

WHICH THE RECORD DATA ARE TO BE READ.  THIS VALUE IS THE

NUMBER OF BYTES WHICH MTPCKG WILL ENDEAVOR TO READ:  IN NO

CASE WILL MORE THAN BUFLEN BYTES OF DATA BE TRANSMITTED TO

THE BUFFER, BUT IF THE LENGTH OF THE RECORD IS LESS THAN

BUFLEN, THEN ONLY THAT NUMBER OF BYTES WHICH THE RECORD CON-

TAINS WILL BE TRANSMITTED, I.E., THE NUMBER OF BYTES READ

WILL BE MIN(BUFLEN, RECORD-LENGTH).

BUFFER - LOGICAL*1(BUFLEN):  (OUTPUT) THE BUFFER TO RECEIVE THE DATA

FROM THE READ OPERATION.

ACTLEN - INTEGER:  (OUTPUT) THE ACTUAL NUMBER OF BYTES TRANSFERRED

FROM THE TAPE, BUT NOT NECESSARILY INTO THE BUFFER.  IF NO

ERROR OCCURRED, THEN THIS IS THE ACTUAL LENGTH OF THE RECORD.

THE NUMBER OF BYTES OF DATA READ INTO THE BUFFER IS ALWAYS

MIN(BUFLEN, ACTLEN).  IF ACTLEN IS .GT. BUFLEN, THEN THE

BUFFER WILL CONTAIN THE FIRST BUFLEN BYTES OF DATA FROM THE

RECORD.

RESULT - INTEGER:  (OUTPUT) POSSIBLE NON-ZERO VALUES:

RESULT=1 - AN I/O ERROR OCCURRED

RESULT=2 - THE TAPE IS OFFLINE

RESULT=3 - THE RECORD WAS AN END-OF-FILE MARK

RESULT=4 - THE END-OF-TAPE MARK WAS ENCOUNTERED DURING

THE READ OR THE TAPE WAS POSITIONED PAST THE

END-OF-TAPE MARK AT THE START OF THE READ.

RESULT=3 TAKES PRECEDENCE OVER RESULT=4 WHEN

BOTH SITUATIONS OCCUR TOGETHER.

RESULT=5 - NO START OF RECORD WAS ENCOUNTERED WITHIN

SEVERAL FEET OF FORWARD MOTION OF THE TAPE.

PRESUMABLY THE READ WAS INITIATED WHEN THE

TAPE WAS POSITIONED PAST THE PHYSICAL END OF

DATA.

CALL MTRDBK(ID, BUFLEN, BUFFER, ACTLEN, RESULT) -- TO READ THE PREVIOUS

===============================================    RECORD IN THE REVERSE

DIRECTION FROM THE CURRENTLY LOADED TAPE INTO A BUFFER.

BUFLEN - INTEGER:  THE LENGTH OF THE BUFFER, AS ABOVE.

BUFFER - LOGICAL*1(BUFLEN):  (OUTPUT) THE BUFFER TO RECEIVE THE DATA.

ACTLEN - INTEGER:  (OUTPUT) THE ACTUAL NUMBER OF BYTES TRANSFERRED,

AS ABOVE.  IF ACTLEN IS .GT. BUFLEN, THEN THE BUFFER WILL

CONTAIN THE LAST BUFLEN BYTES OF DATA FROM THE RECORD.

RESULT - INTEGER:  (OUTPUT) POSSIBLE NON-ZERO VALUES:

RESULT=1 - AN I/O ERROR OCCURRED

RESULT=2 - THE TAPE IS OFFLINE

RESULT=3 - THE RECORD WAS AN END-OF-FILE MARK

RESULT=4 - TAPE WAS POSITIONED AT BEGINNING-OF-TAPE BE-

FORE THE READ.

RESULT=5 - NO RECORD ENCOUNTERED WITHIN SEVERAL FEET OF

BACKWARD TAPE MOTION.

CALL MTWRIT(ID, BUFLEN, BUFFER, ACTLEN, RESULT) -- TO WRITE A RECORD ON

===============================================    THE CURRENTLY LOADED

TAPE FROM A BUFFER.

BUFLEN - INTEGER:  THE LENGTH OF THE BUFFER, E.G., THE NUMBER OF

BYTES TO BE WRITTEN.  BUFLEN MUST BE .GE. 14.

BUFFER - LOGICAL*1(BUFLEN):  THE BUFFER CONTAINING THE BUFLEN BYTES

OF DATA TO BE WRITTEN.

ACTLEN - INTEGER:  (OUTPUT) THE ACTUAL NUMBER OF BYTES WRITTEN;  IF

RESULT=0, THEN ACTLEN WILL EQUAL BUFLEN.

RESULT - INTEGER:  (OUTPUT) POSSIBLE NON-ZERO VALUES:

RESULT=1 - AN I/O ERROR OCCURRED

RESULT=2 - THE TAPE IS OFFLINE

RESULT=3 - THE TAPE IS WRITE PROTECTED

RESULT=4 - END-OF-TAPE:  THE TAPE ENTERED THE END-OF-

TAPE REGION DURING THE WRITE, OR THE TAPE WAS

POSITIONED PAST THE END-OF-TAPE POSITION

BEFORE THE WRITE.

CALL MTWEOF(ID, RESULT) -- TO WRITE AN END-OF-FILE MARK ON THE CURRENTLY

=======================    LOADED TAPE.

RESULT - INTEGER:  (OUTPUT) THE POSSIBLE NON-ZERO VALUES OF RESULT

ARE AS FOR MTWRIT, ABOVE.

CALL MTREWI(ID, RESULT) -- TO REWIND THE CURRENTLY LOADED TAPE.  THE TAPE

=======================    IS LEFT AT BEGINNING-OF-TAPE, I.E., AT THE BE-

GINNING OF THE FIRST FILE.

RESULT - INTEGER:  (OUTPUT) THE POSSIBLE NON-ZERO VALUES:

RESULT=1 - AN I/O ERROR OCCURRED

RESULT=2 - THE TAPE IS OFFLINE.

CALL MTUNLD(ID, RESULT) -- TO REWIND THE CURRENTLY LOADED TAPE AND THEN

=======================    TAKE THE TAPE DRIVE OFFLINE.  THE TAPE DRIVE

CANNOT GO ONLINE WITHOUT SUBSEQUENT MANUAL INTERVENTION.

RESULT - INTEGER:  (OUTPUT) AS FOR MTREWI, ABOVE.

CALL MTSKRE(ID, COUNT, ACTCNT, RESULT) -- TO SKIP OVER A NUMBER OF RECORDS

======================================    OF THE CURRENTLY LOADED TAPE IN

EITHER THE FORWARD OR REVERSE DIRECTION.

COUNT  - INTEGER:  THE NUMBER OF RECORDS TO BE SKIPPED:  FORWARD,

IF .GT. 0, REVERSE, IF .LT. 0.  THE OPERATION IS TERMINATED

IF AN END-OF-FILE MARK IS SKIPPED, IF THE END-OF-TAPE MARK

IS SENSED IN THE FORWARD DIRECTION, OR IF BEGINNING-OF-TAPE

ENCOUNTERED IN THE REVERSE DIRECTION.

ACTCNT - INTEGER:  (OUTPUT) THE ACTUAL NUMBER OF RECORDS SKIPPED, IN-

CLUDING A TERMINATING END-OF-FILE MARK, IF ANY.  ACTCNT

IS ALWAYS .GE. 0.

RESULT - INTEGER:  (OUTPUT) THE POSSIBLE NON-ZERO VALUES ARE AS FOR

MTREAD, ABOVE, WHEN SKIPPING FORWARD;  WHEN SKIPPING IN

REVERSE, THE POSSIBLE VALUES OF RESULT ARE AS FOR MTRDBK,

ABOVE.

CALL MTSKFI(ID, COUNT, ACTCNT, RESULT) -- TO SKIP OVER A NUMBER OF END-OF-

======================================    FILE MARKS ON THE CURRENTLY LOADED

TAPE IN EITHER THE FORWARD OR REVERSE DIRECTION.

COUNT  - INTEGER:  THE NUMBER OF END-OF-FILE MARKS TO BE SKIPPED:

FORWARD, IF .GT. 0, REVERSE, IF .LT. 0.  THE OPERATION IS

TERMINATED BY BEGINNING-OF-TAPE IN THE REVERSE DIRECTION;

THE OPERATION IS NOT TERMINATED BY END-OF-TAPE IN THE FOR-

WARD DIRECTION.  NOTE THAT, FOR SKIPPING IN THE REVERSE DI-

RECTION NOT TERMINATED BY BEGINNING-OF-TAPE, THE TAPE IS

LEFT POSITIONED AT THE END OF A FILE;  IN THE OTHER CASES

THE TAPE IS LEFT POSITIONED AT THE START OF A FILE.

ACTCNT - INTEGER:  (OUTPUT) THE ACTUAL NUMBER OF END-OF-FILE MARKS

SKIPPED.  ACTCNT IS ALWAYS .GE. 0.

RESULT - INTEGER:  (OUTPUT) THE POSSIBLE NON-ZERO VALUES ARE:

RESULT=1 - AN I/O ERROR OCCURRED

RESULT=2 - THE TAPE IS OFFLINE

RESULT=4 - THE END-OF-TAPE MARKER WAS ENCOUNTERED WHILE

SKIPPING FORWARD, OR THE BEGINNING OF TAPE

MARKER WAS ENCOUNTERED WHILE SKIPPING BACKWARDS.

RESULT=5 - NO END-OF-FILE MARK OR DATA RECORD WAS EN-

COUNTERED WITHIN SEVERAL FEET OF TAPE MO-

TION.

NOTE THAT RESULT=3 DOES NOT OCCUR.

CALL MTSETW(ID, EXTERN, RESULT) -- TO SET THE I/O COMPLETION MODE.  THE I/O

===============================    COMPLETION MODE HAS TWO STATES, INTERNAL

AND EXTERNAL.  WHEN THE I/O COMPLETION MODE IS INTERNAL, THEN

EACH OF THE PROCEDURES OF MTPCKG RETURNS TO ITS CALLER ONLY

AFTER ALL OF ITS OPERATIONS HAVE COMPLETED, INCLUDING DATA

TRANSFER AND TAPE MOTION, AT WHICH TIME THE CALLER RECEIVES

THE APPROPRIATE COMPLETION STATUS INFORMATION.  WHEN THE I/O

COMPLETION MODE IS EXTERNAL, THEN THOSE PROCEDURES OF MTPCKG

WHICH INVOLVE TAPE MOTION (EXCEPT MTPREP), I.E., MTREAD, MTRDBK,

MTWRIT, MTWEOF, MTREWI, MTUNLD, MTSKRE AND MTSKFI, RETURN TO

THE CALLER AS SOON AS THE I/O OPERATION HAS BEEN INITIATED;

NO COMPLETION STATUS INFORMATION IS COMMUNICATED TO THE CALLER

AT THAT TIME, VALUES OF ZERO BEING ASSIGNED TO THE OUTPUT PA-

RAMETERS, RESULT, ACTLEN OR ACTCNT.  IN THIS CASE THE CALLER

MUST SUBSEQUENTLY CALL EITHER MTWAIT OR MTCHCK (SEE BELOW),

AND THIS PROCEDURE WILL AWAIT THE COMPLETION OF THE PREVIOUSLY

INITIATED I/O OPERATION, THEN RETURNING THE COMPLETION STATUS

VIA THE OUTPUT PARAMETERS, RESULT AND VALUE.  THE STATE OF THE

I/O COMPLETION MODE HAS NO EFFECT ON THE OPERATION OF OTHER

PROCEDURES OF MTPCKG THAN THOSE LISTED ABOVE.

EXTERN - LOGICAL:  .TRUE. TO SET THE I/O COMPLETION MODE TO EXTERNAL;

.FALSE. TO SET THE I/O COMPLETION MODE TO INTERNAL.  THE

DEFAULT STATE OF THE I/O COMPLETION MODE IS INTERNAL.

RESULT - INTEGER:  (OUTPUT) POSSIBLE NON-ZERO VALUES:

RESULT=1 - THE I/O COMPLETION MODE IS UNCHANGED, I.E.,

THE I/O COMPLETION MODE WAS ALREADY IN THE

STATE SPECIFIED BY THE INPUT PARAMETER, EXTERN.

CALL MTNOOP(ID, VALUE, RESULT) -- TO PERFORM NO OPERATION.  THIS HAS THE

==============================    EFFECT OF INSURING THAT AN IMMEDIATELY

SUCCEEDING CALL OF MTWAIT OR MTCHCK (SEE BELOW) WILL RETURN

WITH THE SPECIFIED "VALUE" AND RESULT = 0.

VALUE  - INTEGER:  THE "VALUE" TO BE RETURNED BY A SUCCEEDING CALL

OF MTWAIT OR MTCHCK.

RESULT - INTEGER:  (OUTPUT) IS ALWAYS ZERO.

CALL MTCHCK(ID, VALUE, RESULT) -- IS AN ALIAS FOR CALL MTWAIT, BELOW, Q.V..

==============================    THIS ENTRY IS INCLUDED IN MTPCKG ONLY

BECAUSE IT WOULD SEEM MORE PERSPICUOUS THAN MTWAIT TO USE AFTER

OPERATIONS WHICH DO NOT INVOLVE TAPE MOTION OR WHEN THE I/O

COMPLETION MODE IS INTERNAL.

VALUE  - INTEGER:  (OUTPUT) SEE MTWAIT, BELOW.

RESULT - INTEGER:  (OUTPUT) SEE MTWAIT, BELOW.

CALL MTWAIT(ID, VALUE, RESULT) -- TO AWAIT THE COMPLETION OF THE MOST RE-

==============================    CENTLY INITIATED I/O OPERATION OF MTPCKG

(WITH THE SAME ID) AND TO OBTAIN ITS COMPLETION STATUS.  THIS

OPERATION INVOLVES A PHYSICAL WAIT ONLY FOR I/O OPERATIONS WHICH

ENTAIL TAPE MOTION.  THE OPERATION MTCHCK, ABOVE, IS IDENTICAL

TO MTWAIT.  THESE FUNCTIONS MAY BE CALLED NO MATTER WHAT THE

STATE OF THE I/O COMPLETION MODE, BUT THEIR CALLING IS MANDATORY

WHEN THE I/O COMPLETION MODE IS EXTERNAL AND THE IMMEDIATELY

PRECEDING CALL OF MTPCKG WAS TO MTREAD, MTRDBK, MTWRIT, MTWEOF,

MTREWI, MTUNLD, MTSKRE OR MTSKFI, EXCEPT WHEN THE NEXT CALL IS

TO MTCLOS, MTWAIT OR MTCHCK.

VALUE  - INTEGER:  (OUTPUT) RECEIVES THE VALUE THAT IN INTERNAL I/O

COMPLETION MODE WOULD HAVE BEEN RETURNED TO THE CALLER VIA

THE OUTPUT PARAMETER ACTLEN OR ACTCNT.  THE VALUE ZERO IS

RETURNED IF THE PRECEDING I/O OPERATION WOULD NOT HAVE RE-

TURNED SUCH A VALUE (E.G., MTWEOF).

RESULT - INTEGER:  (OUTPUT) RECEIVES THE VALUE THAT IN INTERNAL I/O

COMPLETION MODE WOULD HAVE BEEN RETURNED TO THE CALLER VIA

THE OUTPUT PARAMETER RESULT.

CALLS OF MTWAIT (OR MTCHCK) DO NOT RESET THE STORED VALUES FOR THE

OUTPUT PARAMETERS, VALUE AND RESULT, SO THAT SEVERAL CALLS OF MTWAIT

WILL ALL RETURN THE SAME RESULTS PROVIDING THAT NO OTHER CALLS OF

MTPCKG WITH THE SAME ID INTERVENE.

THE TABLE GIVEN BELOW SUMMARIZES THE REQUIREMENTS FOR CALLING MTWAIT

WHEN THE I/O COMPLETION MODE IS EXTERNAL.  THE ENTRIES IN THE (A, B)

POSITIONS OF THE TABLE SIGNIFY AS FOLLOWS:

* :  NOT APPLICABLE;

+ :  A CALL OF MTWAIT OR MTCHCK IS MANDATORY BETWEEN A CALL

OF A AND A CALL OF B;

- :  A CALL OF MTWAIT OR MTCHCK IS NOT REQUIRED BETWEEN A CALL

OF A AND A CALL OF B.

\     O | C | W | C | N | S | P | S | S | N | R | R | W | W | R | U | S | S |

\B   P | L | A | H | A | E | R | E | E | O | E | D | R | E | E | N | K | K |

A\   E | O | I | C | M | N | E | R | T | O | A | B | I | O | W | L | R | F |

\  N | S | T | K | E | S | P | M | W | P | D | K | T | F | I | D | E | I |

|---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

OPEN| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

CLOS| * | * | * | * | * | * | * | * | * | * | * | * | * | * | * | * | * | * |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

WAIT| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

CHCK| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

NAME| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

SENS| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

PREP| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

SERM| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

SETW| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

NOOP| * | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

READ| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

RDBK| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

WRIT| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

WEOF| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

REWI| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

UNLD| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

SKRE| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|

SKFI| * | - | - | - | + | + | + | + | + | + | + | + | + | + | + | + | + | + |

----+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---+---|