Powder Diffraction Program

Information Center

Leiden, The Netherlands

Program Questionnaire

1. Program:

Name: SBGBBG

Function: SBGBBG evaluates the full strain or stress tensor of a

polycrystalline material from X-ray reflection positions.  To obtain the

strain (stress) tensor, the user has to provide the reflection positions

obtained from an X-ray diffraction measurement, according to the well

known PHI-PSI-procedure.  Furthermore, the user has to provide either

the strain free (ideal) reflection positions or the strain free (ideal)

lattice constants, together with the crystal system and the reflection

indices.  The accuracy of the result depends strongly on the correctness

of the strain free values.  To obtain the stress tensor, the user also

has to provide the X-ray elastic constants 1/2S2(HKL) and S1(HKL).

Since the strain (stress) tensor has 6 independent components, one has

to measure at least 6 with respect to PHI and PSI different reflection

positions.  Each measured reflection results in one equation for the

evaluation of the 6 unknown tensor components.  All measurements

together form an equation system solved in SBGBBG by a least squares

procedure.  Thus it is necessary to do measurements at different PHIs

and PSIs to receive linear independent equations.

SBGBBG has a simple interactive interface to facilitate the input when

one uses SBGBBG for the first time.  In routine work it is more

convenient to pass data to SBGBBG via an input file.

SBGBBG does also some other minor evaluations, namely (a) correction of

the lattice constants for cubic materials by the use of extrapolation

functions, (b) calculation of hexagonal lattice constants from measured

reflection positions by a least squares procedure, (c) calculation of

crystallite size and microstrain from Scherrer's equation, (d) data

preparation for the graphical solution of the mentioned equation system

by Doelle's method.

Therefore, SBGBBG needs a lot of data (reflection position, FWHM, (HKL)

and so on) and it demands them.  Data which do not belong to the

evaluation of the strain tensor, can be replaced by dummy values (not

too unreasonable ones because of some input check) without harm for the

tensor calculation.  For example, if one has no FWHMs, then one must

provide some dummy values for them, but then one may not calculate

crystallite size and microstrain.

2. Author

Name(s): Thomas Wieder

Address: University Kassel

Department of Physics

Experimentalphysik III

Wilhelmshoeher Allee 73

D-34109 Kassel

Germany

State: Hessen

Country: Germany

Zip Code: D-34109 Kassel

Tel.: +49-561-8046437

Fax.: +49-561-8046327

Telex:

E-mail: WIEDER@PHYSIK.UNI-KASSEL.DE

3. Organization of financial support:

4. State of the program: Ready for distribution

5.  Demonstration version available: No, only the full version is available

6. Type of assistance in case of problems: Program Author support

7. Is reference on use obligatory? Yes

8. Software and hardware requirements:

Program Language: Fortran 77

Compiler Type: Norsk Data Fortran 500 Version 210190K02, Microsoft

Fortran Version V5.0 1989, Siemens FOR1 Version V2.0A00 1988, IBM AIX XL

Fortran Compiler /6000, VAX Fortran.

Version: See the date of last modification.

Computer Type on which the program runs:

Mainframes: VAX/VMS, Siemens

Workstations: Norsk Data, IBM AIX

Personal Computer: IBM Compatible

9. Special techniques and/or necessary additional software

required for the installation: none

10. Installation Reference:

11. Will source code be supplied: Yes

12. Distribution Format: Source

13. Would current and updated code be supplied to the central

distribution site of the Dutch Association of Crystallographers:

Yes

13a. Computers on which the program has been used successfully:

Minimum hardware requirements:

Memory size: 1 MB

Coprocessor needed: no

(the compiled version distributed with the source code does need a

coprocessor)

Graphics Software Package needed: none

Graphics Hardware needed: none

14. Program Category: Crystallinity/Strain/Texture

15.  Form of documentation: machine readable.

16. Full literature references to program and its uses (if any):

17.  Instructions for obtaining a copy of the program including any

costs involved and the type of the medium for the copy :

No fee is requested.  Write or E-mail to the author.  If the program is

to be sent via a floppy, then please provide a floppy for an

IBM-compatible PC.  Tell me the operating system of your computer.

Available via: E-mail

Floppy Size: 3 1/2 DS, HD or 51/4 DS, HD

FTP

18. Sources for versions of the program which have been modified to run

on computers other than the author's: none

19.  Support will be provided by the author: Yes

20. Program abstract:

21. Additional comments (note item number):

To distribute SBGBBG, the following files should be included:

SBGBBG.DOC = program manual, readable with any Ascii editor

SBGBBG.FOR = the source code

P1E3.DAT = an example input file

P1E3.OUT = an example output file

CHA67.DAT = an example input file

CHA67.OUT = an example output file

For the first installation, try to compile SBGBBG.FOR without any source

code modifications.  The compilation of SBGBBG should not provoke error

messages, but result in an executable file.

If you use a personal computer, then the source code in file SBGBBG.FOR

may be too long for your compiler.  Then spilt SBGBBG.FOR into two (or

more) files of equal size named SBGBBG1.FOR and SBGBBG2.FOR, compile and

link them.

Two problems could arise: (a) during linking the linker could report an

unresolved reference, (b) when running SBGBBG it could happen that the

file names used by SBGBBG do not fit to the operating system of your

computer.  In the following, these problems are explained and the way to

remove them is described.

SBGBBG contains one operating system dependent subroutine.  The purpose

of this subroutine (named BEFEHL) is to allow a call of the operating

system's commands without leaving SBGBBG.  The correct "system command

call" is enabled by informing SBGBBG about the running operating system.

The Fortran OPEN command for file opening has been used in a way that it

should work on most common computers without need of change.

Nevertheless the file names are often incompatible between different

operating systems.  SBGBBG "knows" the appropriate file name conventions

according to several operating systems (VAX VMS, Norsk Data Sintran,

Microsoft DOS, IBM AIX (UNIX)).

To tell SBGBBG which operating "system command call" and which file name

convention is to be used, do the following before compiling: enable the

source code line

C      SYSTEM='SINTRAN'

C      SYSTEM='DOS'

C      SYSTEM='VAX'

C      SYSTEM='UNIX'

containing the appropriate system name at the beginning of SBGBBG's

source code by removing the "C" in the first column of the line (and let

the other ones be disabled by the "C" in their first columns).  For

example, to enable UNIX it has to be

C      SYSTEM='SINTRAN'

C      SYSTEM='DOS'

C      SYSTEM='VAX'

SYSTEM='UNIX'

One of these conventions will be enabled by default and it should be the

correct one for your computer, if you have told me your operating

system.

LEGAL MATTERS

I make no warranties, express or implied, that the program SBGBBG is

free of error, or is consistent with any particular standard of

merchantability, or that it will meet your requirements for any

particular application.  It (SBGBBG) should not be relied on for solving

a problem whose incorrect solution could result in injury to a person or

loss of property.  If you do use the program in such a manner, it is at

your risk.  The author disclaims all liability for direct or

consequential damages resulting from your use of the program.