@@@@@@         @@@@@@         @@@@@@         @@@@@@

@@@@@@@@       @@@@@@@@       @@@@@@@@       @@@@@@@@

@@@    @@@     @@@    @@@     @@@    @@@     @@@    @@@

@@@            @@@    @@@     @@@    @@@      @@@   @@@

@@@            @@@    @@@     @@@    @@@       @@@  @@@

@@@            @@@    @@@     @@@    @@@        @@@ @@@

@@@            @@@@@@@@@@     @@@    @@@     @@@ @@@

@@@            @@@@@@@@@@     @@@    @@@     @@@  @@@

@@@            @@@@@@@@@@     @@@    @@@     @@@   @@@

@@@    @@@     @@@    @@@     @@@    @@@     @@@    @@@

@@@@@@@@      @@@    @@@      @@@@@@@@       @@@@@@@@

@@@@@@       @@@    @@@       @@@@@@         @@@@@@

Crystal  Analysis  Operating  System

***    C. A. O. S. ***

(vers 2099)

U S E R   M A N U A L

by

Camalli M., Capitani D., Cerrini S., Spagna R.

Istituto di Strutturistica Chimica “G.Giacomello” - CNR.

Area della Ricerca di Roma

C.P. 10

00016   Monterotondo Stazione

(Roma)   Italy

Cascarano G., Giacovazzo C.

Istituto per Sviluppo di Metodologie Cristalografiche - CNR and Dipartimento Geomineralogico

Università di Bari

Campus Universitario

Via Orabona 4

70125    Bari

Italy

Italian Patent No. 35403c/86. CAOS user manual, Istituto di Strutturistica Chimica CNR.

## Some general information

The input necessary for all jobs consists of a sequence of commands, data and directives which determine the course that the calculation should take. The commands are headed by '#’ or ‘%’ character, directives and data follow the related command.

CAOS recognises the following commands:

#INITIALIZE 			Initialize the direct access file

#FILE                  binary_file	give the file name of the direct access binary file

#STRUCTURE    binary_file	like  #FILE

#TITLE                title		give the title of the structure

#JOB                   title		like  #TITLE

#INPUT               input_file	give the input file name

#OUTPUT           output_file	give the output file name

#READ                list_no	read the data of the list_no list

#LIST                  list_no	like  #READ

#PRINT               list_no	print the data of the list_no list

#UPDATE            list_no	update the list_no list

#DUMP                list_no	dump on requested file the list_no list

#PUBLICATION   list_no	print the list_no list as tables for publication

#DATA 			read reflns and obtain set of unique reflections put on Wilson scale

#SAVE				save list 5 and list 12 as saved lists

#RESTORE			restore the saved lists 5 and 12

#SORTING			sort the reflections by h,k,l

#MERGE			merge equivalent reflections

#NORMAL			calculate Wilson plot

#FOURIER			calculate a Fourier map

#PATTERSON			calculate a Patterson map

#SEARCH			calculate connectivity from list of atoms (list 5) and produce a list 35

#LSQ				Least Squares routine

#AGREEMENT			analysis of Fo Fc

#DISTANCE			calculate distances

#ANGLE			calculate angles

#SHORT			calculate short distances

#LONG				calculate long distances

#HYDROGEN			calculate geometrical positions of H atoms

#GEOMETRY			geometrical calculation routine

#PLOT				plot the molecule on screen or on HP plotter

#INFO				give information about the structure data

#CIF				produce a file CIF

#END				End of the program

All commands and directives are in free format (between columns 1-80) and are case independent. Only the first four characters are significant. If the first character following the header character ‘#’ is a blank character, then the record is interpreted as a comment.

CAOS uses a direct access binary file to store the data of the crystal structure and the name of this file is the first information requested by the program. We remind that the binary file must be indicated without the extension .BIN. The program inserts (or replaces) the extension

The input_file is a text file containing directives and data (created previously with an editor)

The output_file is file containing the results.

Only for Windows95 or NT platform: at the beginning, the program opens a window asking the directory where are stored the files of the structure under examination, the name of the binary_file and the names of input_file and output_file. In the last two questions, if the files are non given, the interactive mode is selected

Only for Unix and OpenVMS platform: to run the program, write the following command:

caos  [binary_file  [input_file   [output_file]]]

All the strings between brackets are optional. If (bynary file) is ommited, no other file can appear in the command line.

If the binary_file is not given, the program prompts:

please enter the binary file name (without extension) !

>

If the input_file is missing, the interactive mode is selected and the output will be on the screen.

If the output_file is not present, the output is on the screen.

## Input/output of lists

The data required to solve a crystal structure must be input to the computer, translated into an internal format and stored on the binary file.

On the disk, different types of data are recorded separately, in what they are called 'LISTS', each list holds only one type of crystallographic information and it is identified by a number, called 'LIST TYPE NUMBER', and by a alpha-numeric string (the first four characters are significant), called IDENTIFYING STRING

LIST TYPE NUMBER  	IDENTIFYING STRING       	TYPE OF DATA

1               	CELL                        	cell parameters

2               	SYMMETRY             	space group symmetry

3               	FORM                 	form scattering factors and cell content

4               	WEIGHTING            	weighting parameters

5               	ATOMS                	atomic parameters

6               	REFLECTIONS          	reflections

12               	DREFINEMENT          	refinement instructions

14               	LIMITS               	fourier directives

15               	PEAK                 	peaks localised from the map

18               	CONDITIONS           	min & max values of the data

35                                    	peaks interpreted by the

program.

Commands to control the lists are:

#READ   (list number or string)

#LIST                 "                     (equivalent to READ)

#PRINT              "

#DUMP              "                     (valid only for 5,6,12,15)

#UPDATE          "                     (valid only for 1,5,6)

#PUBLIC            "                     (valid only for 5,6)

The command #DUMP  ask for the file name to dump the data. In addition, it is possible use the keyword FILE to dump all the lists:

#DUMP   FILE

## Cell parameters	List 1

#READ  CELL

a   b   c          

cell dimensions a, b, and c are in angstroms,     and    in degrees.

#read cell

10.818 9.062 11.822 90 91.87 90

## Space group symmetry	List 2

#READ  SYMMETRY    string

String is the symbol of the space group, according to International Tables (1974). Blanks are necessary among the terms constituting the space group symbol .

#read symm    P  21  21  21

#read  2   P  21/c

## Form scattering factors and Cell content	List 3

#READ   FORM

Species1   n1   Species2    n2    Species3   n3    .........

Keywords

Speciesi is the chemical symbol of atomic type i, ni is the corresponding number of atoms in the unit cell (to a maximum of 8 atomic types). For each chemical  element up to Cf (Z=98) X-ray and electrons scattering factor constants are stored, together with information on the atomic number and weight, covalent and Van-der-Waals radii., etc. in the file SirWare.xen.

The scattering factors are:

f r =  a i  e -bi sin(/)2 + c+ f’          	(real component)

f i = f “ 					(imaginary component)

The structure of the file SirWare.xen is:

(Header record)	06.96    ---- scattering factors ----

(no. of records)  	     9

#   Atomic   Weight    coval_radius    vanderWaals_radius

(x_ray)		#         a(1)    b(1)       a(2)    b(2)    a(3)    b(3)

#                a(4)    b(4)       c

#            f’(Mo)   f”(Mo)    mu/rho

#            f’(Cu)    f”(Cu)    mu/rho

(electr.)		#         a(1)    b(1)       a(2)    b(2)    a(3)    b(3)

#                a(4)    b(4)       c

(neutrons)		#         fact

#     Name_specie

h      1    1.008   .320         1.33

h           .48992  20.65930    .26200   7.74039    .19677  49.55190

h           .04988   2.20159    .00130

h           .00000    .00000    .38000

h           .00000    .00000    .43500

h           .37540  15.49460    .14080   4.12610    .02160    .02460

h          -.10120  46.88400    .00000

h         -0.37200

h  Hydrogen

etc...

Accepted keywords:

FILE    file name 		containing the information of the atomic species (default = SirWare.xen)

WAVE   (value or code)    		value of the  or symbol (Mo and Cu)

DIFFRACTION  (code)     		code indicate the kind of data:

x-ray

electron

neutron

ANOMALOUS    species  f’   f”    	give the anomalous dispersion factors for the atomic species if different from what is stored in the SirWare.xen file.

SFACTOR   specie a(1)  b(1)  a(2)  b(2)  a(3)  b(3)  a(4)  b(4)  c  f’   f”  mu/rho  r  wt

(like SHELX instruction)

UNIT           n1  n2  n3  …		number of atoms of each type in the unit cell, in the directive SFACTOR order, (like SHELX instruction)

END	ends the command.

#read form

hg 1 p 2 c 15 h 22

end

#read  3

o 1  c 21   h 24

wave  cu

end

## Weighting scheme	List 4

#READ   WEIGHTING

type   npar  par1   par2  ...

type    		number of the weighting scheme requested (1 - 18)

npar		number of parameters (drefaulted)

par1  par2 ...	npar  parameters of the weighting scheme (defaulted)

The available weighting schemes are the following :

1	w = Fo/p(1)  if Fo   p(1)

w = p(1)/Fo  if Fo  p(1)

default ; p(1)=4*Fo(min)

2	w = 1/(p(1)+p(2)*Fo+p(3)*Fo**2)

default: p(1)= 2*Fo(min); p(2)=1.0; p(3)=2./Fo(max)

3	w = (sin()/)**p(1)

default: p(1)=1.0

4 	w = (/sin())**p(1)

default: p(1)=1.0

5	w = p(1)/((Fo ))**2

6	w = p(1)

default ; p(1)=1.0

7 	w = 1/(1.+((Fo-p(2))/p(1))**2 )

default: p(1)= Fo(max); p(2)=Fo(min)

8	w = 1.0		if Fo    p(1)

w = p(1)/Fo	if Fo    p(1)

default ; p(1)=4*Fo(min)

9 	w = p(1)*(Fo)

10	w = p(1)/(Fo)

11	w = (sin()/)**p(4) / (p(1)+p(2)*Fo+p(3)*Fo**2)

default: p(4)=1.0; p(1)= 2*Fo(min); p(2)=1.0; p(3)=2./Fo(max)

12 	w = 1./( p(1)*(Fo )**2+p(2)*P+p(3)*P**2)

where P=(Fo+2Fc)/3   (shelx scheme)

default: p(1)=1.0; p(2)=2.*P(min); p(3)=2./P(max)

13	w = 1/ (p(1)+p(2)*Fo+p(3)*Fo**2+p(4)*(sin()/))

default: p(1)= 2*Fo(min); p(2)=1.0; p(3)=2./Fo(max); p(4)=1.0

14	w = 1/(p(1)+p(2)*Fo+p(3)*Fo**2)

default: p(1)= 2*Fo(min); p(2)=1.0; p(3)=2./Fo(max)

as scheme 2, but #WEIGHT uses different algorithm to find the best coefficients

15	w = 1/(p(1)+p(2)*(sin()/)+p(3)*(sin()/)**2)

default: p(1)= (sin()/)(min); p(2)=1.0; p(3)=(sin()/)(max)

16	w = 1/[aiT*i(F)]

no default, the Chebyshev coefficients ai must be calculated using #WEIGHT

17	w = (sin()/)**p(3) / (p(1)*(Fo )**2+p(2)*Fo**2)

default: p(3)=1.0; p(1)=1.0; p(2)=2./Fo(max)

18	w = 1/((p(4)*(sin()/)**p(5)+ p(1)+p(2)*Fo+p(3)*Fo**2)*(Fo )**2)

default: p(4)=1.0; p(5)=1.0; p(1)= 2*Fo(min); p(2)=1.0; p(3)=2./Fo(max)

The weight is multiplied by a scale factor (SC) to achive:

w2 /(Nref-nparam) = 1.0

The value of SC is set = 1.0 when the weighting scheme is input.

#read weight

2

(Cruickshank weighting scheme using default values)

## Atomic parameters	List 5

The available parameters of the atoms obtained are stored in the direct access file as following:

Species     Serial_number    x    y    z    Thermal_factor     Occupation

where Species is the chemical symbol of the atom and it must be given in the list 3; Serial_number is a sequence number of that species; the Thermal_factor is B(iso) for an isotropic atom (the exponential factor takes the form exp[B(sin()/)2] and the Thermal_factors are  (11  12  13  22  23  33 ) for an anisotropic atom (the exponential factor takes the form exp(-ijhihj ). (in the test b)

#READ   ATOMS

Scale  B(overall)  Extinction  Polarity                            scale to be applied to Fc; B(overall) is used in the refinement block-diagonal approximation, coeff. of extiction, coeff. of polarity

The record(s) for each atom are the following:

- Species  serial   x   y   z    [b(iso)]   [(occ)]  [(idev)]  [(imix)]  [(name)]

if b(iso)=0,  read a record containing the b(i,j):

- Species    b(11)  b(12)  b(13)  b(22)  b(23)  b(33)

if idev=1, read a record containing the standard deviations on the parameters :

- Species    (x)   (y)   (z)   (b(iso))   (occ)

if b(iso)=0, read the a new record containing the standard deviations on b(i,j):  :

- Species   (b(11)) (b(12)) (b(13)) (b(22)) (b(23)) (b(33))

if imix>0, read the a new record containing Species & occ for imix times

- Species1   occ1   Species2   occ2   …

If b(iso) is not present, the default value is B(overall); default value for occ is 1.00; Idev and imix if not present assume the value 0, the maximum value for imix is 4; the name is a string identifying the atom (max 8 characters), if it is not present, a string will be built Species(serial).

The sequence of atoms can ends with the connectivity :

nbond 	number of bonds

n1   n2   code  	for each bond : n1 and n2 are the sequence number in the list 5 of the atoms; code is = 0.

The list 5 ends with : ##

If the connectivity is not present, the program builds it calling the interpretation routine.

#read atoms

1.035  5.63  0.00  0.00

o  1   -.99299   -.37159   -.36338    4.33    1.0

....

##

It is possible to read a SHEL-X file of co-ordinates giving an appropriate keyword:

#READ  ATOMS  SHELX

the program prompts the request of file name where the Shel-X co-ordinates are.

Example of Shel-X file (file.ins o file.res):

TITL                                        calix

CELL    .71069     28.181   11.769   15.838   90.000   90.000   90.000

ZERR         4       .005     .005     .005     .005     .005     .005

LATT -1

SYMM -X,-Y,  .50000+Z

SYMM   .50000-X,  .50000+Y,  .50000+Z

SYMM   .50000+X,  .50000-Y,Z

SFAC    C   H   O   CL

UNIT   228 192  32   8

MERG      2

OMIT      4

L.S.      4

LIST      1

FMAP      2

PLAN     25

FVAR    1.0

O1      3   1.09077    .03784    .66737  11.00000    .02816    .06526 =

.04829   -.00486   -.00418   -.00302

O2      3   1.04664    .16432    .78076  11.00000    .01328    .07291 =

.05731   -.00699   -.00204   -.00059

O3      3   1.13441    .21943    .60461  11.00000    .03621    .06610 =

…and so on……..

H48     2    .88080    .19495    .17274  11.00000    .14491

H49     2    .75822    .24202    .94141  11.00000    .15600

H50     2    .78244    .12363    .92712  11.00000    .15600

HKLF      4

END

#PRINT  ATOMS

the following keywords are allowed:

STANDARD		print co-ordinates and standard deviations

FULL			print the b(ij) for anisotropic atoms (default the B(equiv) will be printed)

UIJ			analysis of b(ij)

ORTHOGONAL	give orthogonal co-ordinates of atoms

END			ends the command.

#UPDATE  ATOMS

the allowed keywords are:

RESET					copy the list 35 into the list 5.

The directive MATRIX must follow the ATOM instructions (one or more).

ATOM    (atom)				the (atom) define which atoms are used

MATRIX  a11  a12 ..  a33 t1 t2 t3      	apply the matrix to the atoms indicated before.

ANISO   (atom)		transform the thermal factor of the (atom) atoms from isotropic to anisotropic.

ISO         (atom)			from anisotropic to isotropic.

DELETE  (atom)			delete (atom) atoms from list 5 .

ADD species serial x y z b(iso) occ	add the atom to the list 5

SPECIES	(atom)  species		assign the species to the (atom) atoms

SERIAL	(atom)  serial_no.	assign the serial number to the (atom) atom

HEIGHT	(atom)    a		assign the height a to the (atom) atom

BMEDIUM	a			assign the value a to the B(overall)

SCALE 	a			assign the value a to the Scale factor

BOND		(atom1)   (atom1)	the atoms (atom1) and (atom2) are bonded

NOBOND	(atom1)   (atom1)	the atoms (atom1) and (atom2) are not bonded

CONNECTIVITY			the program builds the connectivity

END					end the command.

#PUBLICATION  ATOMS

the following keywords are allowed:

CIF			write a file in CIF format of atomic parameters;

WORD	write a table using the character TAB to put in columns the co-ordinates (useful if a word processor like WORD will be used);

SKIP   n        	n is no. of initial blank (default n  = 1) ;

NO	the decimal point will not be printed (default : the decimal point is used) ;

COMPRESS  (OFF)  	single spacing (default condition), if the option OFF is present, double spacing is required ;

XFIGURE  n      	n is the number of digits for x, y, z (default n  = 15 ) ;

XDECIMAL  n    	n is the number of decimals for x, y, z (default n = 4 ) depending on st. dev.;

BFIGURE  n      	n is the number of digits for b(i,j) (default n  =  14 ) ;

BDECIMAL  n    	n is the number of decimals for b(i,j) (default n = 4 ) depending on st. dev.;

IFIGURE     n     	n is the number of digits for B(iso) (default n = 12 ) ;

IDECIMAL  n     	n is the number of decimals for B(iso) (default n = 1 ) depending on st. dev.;

BEQUIV          		calculate from b(i,j) the B(equiv) ;

UEQUIV          		calculate from b(i,j) the U(equiv) ;

BIJ             		calculate from b(i,j) the B(i,j) ;

UIJ             		calculate from b(i,j) the U(i,j) .

U			write a table with U(equiv) and a table with the U(ij);

B			write a table with B(equiv) and a table with the B(ij);

NDEVIATION  n	n  is the number of digits of the standard deviations;

FIXED			the number of decimals is fixed,

LIST			list the allowed keywords.

END			ends the command.

Examples:

...

#update  5

aniso

end

...

#update  5

aniso  c o

end

...

#update  5

atom sn

matrix    -1 0 0    0 -1 0    0 0 -1   0  0  0

end

#publ  5

end

#publ  5

b

fixed

end

## Reflections 		List 6

#READ  REFLECTIONS

the following keywords are allowed:

UNIT	file_name		 the file where the reflections are stored

FOSQUARE			 the Fo2’s are read and the Fo are calculated and stored on binary file

CARD	 m  n			 m is number of reflections for each record

.n is the number of data to be read (default n=7)

SEQUENCE	(codes..)	which data are read. Codes are:

H   K   L  FO   SIGMA   FC   PHASE   CODE

FORMAT      format	format to be used (remind: the indices are read as floating point numbers)

RADIANT	the phase are expressed in radians

##### END				ends the command.

If the keyword UNIT is not given, the reflections follow the END. The default quantities read in free format for each reflections are:

h    k    l     Fo       (Fo)      Fc      Phase

and it is not necessary that all the information are present. The dummy reflections ends the list.

127  127   127

The list 18 is automatically created.

#PRINT  REFLECTIONS

the allowed keywords are:

ALL			print all the reflections (default).

FIRST      n    		print the first n reflections.

LAST       n    		print the last n reflections.

SCALE       		print the Fo on absolute scale.

COLUMNS    n		number of columns.

LINES           n 		number of rows per page

STATISTIC   		print only some information on list 6.

END			ends the command.

#UPDATE  REFLECTIONS

the allowed keywords are:

CHOP              a	flag as unobserved the  Fo<a*(Fo).

SYSTEMATIC   n 	delete the systematic absences and assign the same indices to all the equivalent reflections. If n=0  applies the centre of symmetry, if n=1 , the centro-symmetric reflections are left as they are.

MATRIX a11 a12 ..  a33  t1 t2 t3  	apply the operator to the indices :

H’ = (H*a11)+(K*a12)+(L*a13)+t1

K’ = (H*a21)+(K*a22)+(L*a23)+t2

L’ = (H*a31)+(K*a23)+(L*a33)+t3

CONDITIONS	the quantities of each reflection are compared with the limit stored in list 18 and not more used in the subsequent calculations. The reflections can be reused changing the limits.

SORTING   ilent    ivelox   	are the code of the sorting

OBSERVED                  	the unobserved reflections are definitively delete from the list 6.

SCALE          a                	F(obs) are multiplied by  a .

MODIFY                    	calculate the equivalent indices and preserve hkl in the limits of list 18.

GENERATE                  	Generate a file with all the equivalent reflection without the centre of symmetry.

END	ends the command

#PUBLICATION  REFLECTIONS

the allowed keywords are:

CIF	write a file in CIF format ;

LASER	write a file in a format according with the laser printer;

COLUMNS    n      	no. of columns per page (default = 4) ;

LINES           n      	no. of lines per page (default = 57) ;

SPACES       n      	no. of spaces between columns (default = 3) ;

SKIP             n  	    	n is no. of initial blank (default n  = 10) ;

SCALE          a       	a multiplies Fo and Fc (default = 1.0 ) .

END	ends the command.

The reflections must be sorted by indices (use #SORTING   )

Example:

#read reflections

end

0 0 1 372.9

0 0 2 387.7

0 0 3 1224.2

0 0 4 640.3

...

12 4 1 29.2

13 1 -1 27.6

13 1 0 61.2

127 127 127

#print 18

#end..

#print  6

statis

end

#end

## Refinement directives	List 12

#READ DREFIN

The following directives define the type of matrix to accumulate the least squares derivatives.  Two type of directives exist, type A and type B ; the first directive must be of type A and only the directive BLOCK can be repeated to start the user-defined blocks approximation.

The label of an atom is Species(Serial_number). The atoms involved into the refinement process can be indicated as:

1) no indication (all the atoms are considered);

2) Species (all the atoms belonging to that species are considered)

3) Label (only that atom is considered)

4) Label1 until Label2 (the atoms in the list starting from Label1 until Label2 are considered)

A mixed way can be used. Examples:

c      			(only carbon atoms are indicated)

c  o  			(carbon and oxygen atoms are indicated)

c(1) 			(only this atoms is indicated)

c(1) until c(5) 	(if c(2), c(3) and c(4) are present in the list between c(1) and c(5), all these atoms are considered)

c(1)  c(12)  		(only c(1) and c(12) are considered)

c(2) c(7) until c(15)    	(the atoms c(2) and c(7),c(8),c(9)...,c(15) are considered)

To indicate the parameters, the following codes are used:

nothing			x y z and b[iso] for isotropic atom

x y z and ijfor anisotropic atom

x  y  z 			positional co-ordinates

x's     			to indicate all three co-ordinates x, y and z

b[iso]    		to indicate the isotropic thermal parameter

11  12  13  22  23  33  	ij anisotropic thermal parameters

ij  ii  			to indicate the off- and the diagonal thermal parameters

occ			occupation (the main species for mixed site)

occ1			occupation of 1st species for mixed site

occ2			occupation of 2nd species for mixed site

occ3			occupation of 3rd species for mixed site

occ4			occupation of 4th species for mixed site

scale	  		overall scale

boverall		the overall temperature factor

extinction		extinction coefficient

polarity 		polarity coefficient

Type A:

FULLMATRIX       atoms    params	The parameters of the indicated atoms are included in a full matrix (the scale factor is automatically included). In the default way, all the atoms are included.

DIAGONAL       atoms   params	The parameters of the indicated atoms are refined using diagonal matrix approximation, one block per atom ; an extra block is added for the scale factor and the overall temperature factor. In the default way, all the atoms are included.

BLOCK             atoms   params	The parameters of the indicated atoms are included in the same block. The directive BLOCK has to be repeated for each block.

DEFAULT    FULL

DEFAULT    DIAG			To reset the refinement conditions.

Type B:

PLUS              atoms    params	The parameters of the indicated atoms are added to those defined by the previous instruction of type A

FIXED  (or FIX)   atoms   params	To fix the parameters of the indicated atoms.

EQUIVALENCE    atom1   param1    atom2   param2	To equivalence parameters of the selected atom(s). If atom2 is omitted, param1 and param2 are referred to the atom1 . Using this instruction, param1 is constrained to param2 in the least squares process and only one parameter is refined . This may be due to symmetry or any other constraint process.  The obtained shift, multiplied by proper factor1 and factor2 respectively (see next instruction), is applied to param1 and param2 . The default value for factor1 and factor2 is 1.00.

FACTOR     atom    param     coef	To set the value of the factor coef which multiplies the shift of the parameter of the indicated atom.

RIDE             atoms1     atoms2	Normally the atoms2 indicates hydrogen atoms and atoms1 one of the species carbon, oxygen, nitrogen and boron if they are present in the content of the cell. The hydrogen co-ordinates 'ride' on the atoms to which they are attached.

DIST       target  sigma atom1  atom2   	The distance between atom1 and atom2 is restrained to the target value with the estimated standard deviation sigma.

ANGLE   target  sigma  atom1  atom2  atom3	The angle is restrained to the target value with the e.s.d. sigma. The angle is defined as the angle subtended at atom2 by atom1 and atom3.

PLANAR  [sigma]  atoms	The program computes the mean plane trough the atoms and then restrains each of the atoms to lie in the plane (default value for sigma=0.001Å).

SUM        [sigma]  [atoms]  params	This restraint holds the sum of the parameters. This is the default case for polar space groups, where the singularity up the polar axis may be removed by holding the sum of all the co-ordinates up that axis constant (default value for sigma=0.00001)

END					To end the directives input.

Example:

#read drefin

default   full

end

#read drefin

diag  o  c

ride  c h

##

...

#read drefin

full  br(1) until c(15)  x's  ij

fix br(1) z

ride  c h

end

...

#read drefin

block first until last x's

block o c ij

plus scale

##

...

#read drefin

block o

block c

plus scale

ride c h

end

...

#read drefin

block o c

plus scale

equiv o x y

end

...

#read drefin

full

plus  c(1)   occ

equiv  c(1) occ c(11) occ

weight  c(11)   occ    -1.0

end

...

#read drefin

block c(1) until c(48) x's ij

plus h(1) until h(483) x's

plus scale

ride    c(1) until c(48) h(1) until h(483)

block c(101) until c(148) x's ij

plus h(11) until h(1483) x's

ride c(101) until c(148) h(11) until h(1483)

end

...

#read drefin

diag br(1) until c(12)

ride  c h

end

#list 12

full

fix   om1(1)   y

fix   om1(1)    23

fix   om1(1)    12

fix   o(6)     y

fix   o(6)      23

fix   o(6)      12

fix   mgp2(1)  x   z

fix   mgp2(1)   23

fix   mgp2(1)   12

fix   mgp2(2)  x   z

fix   mgp2(2)   23

fix   mgp2(2)   12

fix   mgp2(3)  x y z

fix   mgp2(3)   23

fix   mgp2(3)   12

fix   nap1(1)  x   z

fix   nap1(1)   23

fix   nap1(1)   12

fix   nap1(2)  x y z

fix   nap1(2)   23

fix   nap1(2)   12

fix   nap1(3)  y

fix   nap1(3)   23

fix   nap1(3)   12

fix   nap1(4)  x   z

fix   nap1(4)   23

fix   nap1(4)   12

fix   h(1)     y

fix   fep2(3)  x   z

ride  o h

fix h b[iso]

fix o(1) ij

plus o(1) occ occ1

sum o(1) occ  occ1

fix o(2) ij

plus o(2) occ occ1

sum o(2) occ  occ1

fix om1(1) ij

plus om1(1) occ occ1

sum om1(1) occ  occ1

fix o(3) ij

plus o(3) occ occ1

sum o(3) occ  occ1

fix o(4) ij

plus o(4) occ occ1

sum o(4) occ  occ1

fix o(5) ij

plus o(5) occ occ1

sum o(5) occ  occ1

fix o(6) ij

plus o(6) occ occ1

sum o(6) occ  occ1

fix al(1) ij

plus al(1) occ occ1  occ2   occ3

sum al(1) occ  occ1  occ2  occ3

fix si(2) ij

plus si(2) occ occ1

sum si(2) occ  occ1

fix mgp2(1) ij

plus mgp2(1) occ occ1

sum mgp2(1) occ  occ1

fix mgp2(2) ij

plus mgp2(2) occ occ1  occ2

sum mgp2(2) occ  occ1  occ2

fix mgp2(3) ij

plus mgp2(3) occ occ1

sum mgp2(3) occ  occ1

fix nap1(1) ij

plus nap1(1) occ occ1

sum nap1(1) occ  occ1

end

#end

## Map limits	List 14

The list 14 gives information to calculate a map.

#READ LIMITS

The allowed keywords are:

X  Min Max Step Div   	minimum, max, range and unit for the a axis

Y Min Max Step Div    	for the b axis;

Z Min Max Step Div    	for the c axis .

PEAK   n               	Number of peaks requested

LEVEL   a              	Threshold for peak search routine;

MULTIPLY   a            	a is the scale for electronic density. The program multiplies in any case for 10.0

F000     a             	give the  F(000)   (default : a= 0.0).

ORIENT  ioriz  ivert	the orientation of the map on the printout giving the horizontal axis (ioriz) and the vertical axis (ivert) ;  (remind: 1= axis X ; 2= axis Y ;  3= axis Z) .

DEFAULT              	produce a list 14 using default values.

##### END

Examples:

#list  14

x   0.0 0.5

y  0.0 0.5

z  0.0 1.0

peak  25

level  20.

orient  1 3

end

...

#list 14

default

peak   50

end

...

#list  14

x  0  50  2  100

y  0  50  2  100

z  0 100  2  100

orient   1 3

end

#list 14

default    diag

end

## Reflections conditions		List 18

The list 18 has two functions: 1) to keep the min and max values of some quantities of the reflections; 2) to modify the reflections to be used in the calculations changing the min and max values and updating the list 6.

#READ  CONDITIONS

The allowed keywords are:

H		min	max		min and max values for h ;

K 		min	max 		min and max values for k ;

L		min	max 		min and max values for l ;

FO 		min	max		min and max values for Fo;

SIN 		min	max		min and max values for sin(/)2 ;

FC		min	max		min and max values for Fc ;

F/FC 		min	max		min and max values for Fo/Fc ;

R		min	max 		min and max values for [Fo-Fc]/Fo ;

2THETA	min	max 		min and max values for 2;

RESOLUT	min	max 		min and max values for the resolution;

RESET				update the user values using the stored experimental values;.

SET				calculate all the value from the experimental data of list 6.

HKL   a b c d e f 		coefficients for defining validity conditions to accept reflections:

f > H*a + K*b + L*c + d > e

END				ends the command

Examples:

#list 18

set

r  0.0  0.3

end

#update reflecti

conditions

end

#list 18

fo 2.0   300.

end

#update reflec

condit

end

#list 18

reset

end

#update  6

condit

end

## Input file data

#READ FILE  SHELX

the program prompts the request of the first component of the filename for the Shel-X files .ins and .hkl

At the present, the routine is unable to resolve the symmetry cards and they were jumped. The user have to give this information via CAOS directive:

#READ SYMM  [symbol of the S.G.]

## Data reduction

#SORTING   il   iv   ia	where il is the slowest varying index , iv is the fastest index and ia is a code: if ia=1 the sorting is done using the absolute values of the indices.

#MERGE 		average the close equal reflections

#DATA	follow the reflections, like list 6. The results of this command is:

- read and store list 6

- the equivalent reflections are reported with the same indices

- sorting

- merge

- wilson statistic

- put the fo on absolute scale

- create the list 4, 14, 12, 18 using default values.

Examples:

#title dibromo

#list  1

12.268   12.268   9.923  90. 90. 90.

#list  2   p  41

#list  3

br 8 cl 12 o 4 n 8 c 64 h 52

end

#list 6

end

0   0   2        1.68536        6.29010

0   0   3        9.50451        3.09145

0   0   4      195.01208       2.37237

0   0   5        2.48926        9.29041

1   0   6       208.44153     2.53291

.....

.....

18   3   1       5.03256	      19.05841

18   3   2       33.41090        5.80577

18   4   0        5.12315       19.12062

18   4   1        5.14043       19.18509

127 127 127   5.14043       19.18509

#update 6

syst  0

end

#list 18

set

end

#sort

#merge

#list 18

set

end

#end

The following example is equivalent to the above example:

#title  dibromo

#list  1

12.268   12.268   9.923  90. 90. 90.

#list  2   p 41

#list  3

br 8 cl 12 o 4 n 8 c 64 h 52

##

#data

0   0   2        1.68536        6.29010

0   0   3        9.50451        3.09145

0   0   4      195.01208       2.37237

0   0   5        2.48926        9.29041

1   0   6      208.44153      2.53291

.....

18   3   1        5.03256	      19.05841

18   3   2       33.41090        5.80577

18   4   0        5.12315       19.12062

18   4   1        5.14043       19.18509

127 127 127    5.14043       19.18509

#end

## Patterson and Fourier map

The commands allows some keyword to override the value stored in list 14.

#PATTERSON

The allowed keywords are:

AUTOMATIC			automatic solution of a Patterson map.

OBSERVED			use Fo (default conditions)

CALCULATED			use Fc

PRINT				print output the map

LEVEL       a			threshold of the electron density for peak searching

PEAK        n   			number of requested peaks

F000	     a			a is the F(000) to be added

MULTIPLY   a            	a is the scale for electronic density. The program multiplies in any case for 10.0

END				ends the command

#FOURIER

The allowed keywords are:

OBSERVED			use Fo (default conditions)

CALCULATED			use Fc

DIFFERENCES		use Fo-Fc

PRINT	     k			k=1 print the localized peaks; k=2 print the map

LEVEL       a			threshold of the electron density for peak searching

PEAK        n   			number of requested peaks

F000	     a			a is the F(000) to be added

MULTIPLY   a            	a is the scale for electronic density. The program multiplies in any case for 10.0

END				ends the command

Examples:

#patter

automatic       (if the automatic procedure PAUTO has to run)

end

#patt

end

or

#patt     #        (the character # after the directive means to use the default values)

#fourier  #

#fourier

peak 10

level 40

end

#fourier

diff

pe 30

le 5

end

## LSQ routine

The structure factors Fobs are used in the least squares refinement process and it is possible to ignore data with Fobs less than chop_value multiply (Fobs).

A variety of weighting schemes is available and the user can choose a scheme which does not give marked systematic trends with the magnitude of Fobs or of resolution. If the Cruickshank weighting scheme is selected, at the end of the refinement process a routine calculates the best coefficients of the function. In this way, the scheme would give constant <w2> when the set of w2 values is analysed in any significant systematic fashion and a value of the Goodness of Fit equal to the unit.

A damping factor can be set by the user to improve the convergence properties of the refinement.

The atomic parameters are refined using various type of matrix to accumulate the least squares derivatives. It is possible to use any user-defined blocks approximation starting from the full matrix type until the block diagonal per atom (4x4 for isotropic atom and 9x9 for anisotropic atom).

The scale factor is also refined and the constraints due to the space group are automatically taken into account. Nevertheless, the user can fix any parameters and set any equivalence between refined parameters.

If hydrogen atoms are present, their positional co-ordinates ride on the atoms to which they are attached and their isotropic thermal parameters are fixed equal to that of the attached atom.

NUMBER  n			The number of the first calculation for this run.

PRINT				For each reflection the output of structure factor calculation is printed

VERBOSE  n			code to determine the level of printing:

n=0 (default conditions), the values of R% , wR% , S and some other general statistics

n=1 the atomic parameters

n=2 details of the refinement

ANOMALOUS   code  	the anomalous coefficients are taken into account (code = ON) (default condition), code = OFF no anomalous scattering factor is introduced.

CYCLES  n			the number of requested cycles

CALCULATION			only one structure factor calculation is performed

WEIGHTING   type   npar  par1   par2  ... (see #LIST 4)

type    	number of the weighting scheme requested (1 - 12)

npar	number of parameters

par1  par2 ...	npar  parameters of the weighting scheme

CUTOFF    chop_value		To ignore data with Fo less than chop_value multiplied by (Fo).

CHOP        chop_value	like CUTOFF

DAMPING   damp	The calculated shifts are multiplied by damp before being added to the parameters

SCALE  			To refine the scale only.

ISO     atoms	The thermal factors of the atoms here indicated are transformed into isotropic ones

ANISO   atoms	The thermal factors of the atoms here indicated are transformed into anisotropic ones

NOCHECK			do not check automatically the list 12

Examples:

To run a least squares refinement:

#lsq

end        or

#lsq   #

the run ends when convergence is reached.

#lsq

cycle 4

end

only 4 cycles are requested.

#lsq

scale       (refinement only of the scale)

end

#lsq

calculat    (structure factors calculation only)

end

## Optimized weighting function

(only Cruickshank scheme)

#weight  #

#weight

verbose

table

end

## Agreement analysis

#agree  #

#agree

verbose

print   a        (print the reflections having (Fo-Fc)/Fo > a )

end

## Distances and angles

#DIST       (dmin)  (dmax)                           	calculate the distances between atoms in the range (dmin) and (dmax)

#DIST   (dmin)  (dmax)   (dmin)  (dmax)  	calculate the distances and angles between atoms in the range (dmin) and (dmax)

#SHORT       (dmin) (incr)                             	calculate the distances larger than (dmin) taking into account the atomic radii +/- (incr)(default incr=0.3)

#LONG         (dmin)  (incr)                             	calculate the long distances greater than (dmin) taking into account the van der Walls readii +/- (incr)(default incr=0.3) between atoms of different asymmetric units

#ANGLE     (dmin)    dmax)	calculate ancles between atoms in the range (dmin) and d(max)

## Geometrical calculation of hydrogen atoms

#HYDROGEN

The sequence is :

- define the atom (or the atoms) which the hydrogen atoms are bonded to

- instruction

The allowed keywords are:

33   	(three atoms bonded to carbon atom sp3)

32   	(two atoms bonded to carbon atom sp3)

31   	(one atom  bonded to carbon atom sp3)

22   	(two atoms bonded to carbon atom sp2)

21   	(one atom  bonded to carbon atom sp2)

11   	(one atom  bonded to carbon atom sp1)

AUTOMATIC   	the program finds automatically the hybridization and calculates the hydrogen positions bonded to carbon atoms, nitrogen atoms and boron (default conditions)

SERIAL      n	serial number relative to the first H atoms

NAME    label	species for the generated atoms (default h)

B[ISO]    value	b(iso) for the atoms (default the b(iso) is equal to the bonded atom)

DIST       a 	value of bond distance (default  0.96)

END	ends the command

Example:

#hydrog  #

#hydr

atom  c(1)

33

atom c(11)

atom c(12)

atom  c(13)

32

….

In the first example, the program calculates automatically the positions of H atoms. In the second, the program calculates and adds to the list 5 the co-ordinates of the three hydrogen atoms bonded to c(1) and two H atoms bonded to c(11), the same for c(12) and for c(13) .

## Geometry

It is possible to calculate in this module some geometrical quantities. The esds are estimated from the diagonal elements of the correlation matrix. When the calculation of the coefficients of a plane is requested, they are represented by an identification label given by the user. In this way, subsequent calculations can be done recalling the plane via this label. Furthermore, it is possible to indicate a line passing through two atoms and to compute distances and angles using this vector. For bond lengths and angles calculation, the results can be obtained as a table ready for publication or a Crystallographic Information File file, named structure_name.cif. The torsion angles calculated follow the conventions defined by W. Klyne and V. Prelog, Exper. (1960)16,52.

The atoms involved in the calculations are indicated in a general way:

Species(Serial_number,[Sym,Npl,Tx,Ty,Tz])

where Species and Serial_number are the usual label of the atom; (Sym is the number of the symmetry operator of the space group and the leading sign minus informs that the centrosymmetric operator is requested; Npl is the number of the lattice operator (see table below); Tx, Ty and Tz are positive or negative numbers indicating a lattice translation along axis a, b and c respectively.

The requested atom list in the instructions follows the rules given in LSQ module.

| Lattice | no. of operators | Npl = 1 | Npl = 2 | Npl = 3 | Npl = 4 |
| --- | --- | --- | --- | --- | --- |
| P | 1 | 0 0 0 |  |  |  |
| I | 2 | 0 0 0 | 1/2 1/2 1/2 |  |  |
| R | 3 | 0 0 0 | 1/3 2/3 2/3 | 2/3 1/3 1/3 |  |
| F | 4 | 0 0 0 | 0 1/2 1/2 | 1/2 0 1/2 | 1/2 1/2 0 |
| A | 2 | 0 0 0 | 0 1/2 1/2 |  |  |
| B | 2 | 0 0 0 | 1/2 0 1/2 |  |  |
| C | 2 | 0 0 0 | 1/2 1/2 0 |  |  |

The command is:

#GEOMETRY  STANDARD

One or more of the three following instruction must precede each request of geometrical calculation.

ATOM     atoms		The named atoms are considered

LINE        atom1    atom2	The two atoms identify a straight line

PLANE     label			The label identifies the plane calculated before in the same run.

The following instructions allow the geometrical calculation.

P3P         label			A plane for three atoms is calculated.

PQM       label	A least-squares mean plane is calculated. Deviations of the atoms from the plane are listed.

PNR        label	The coefficients of the plane perpendicular to a line and passing through an atom are calculated

DIST        dmin    dmax	Distances are calculated for all the indicated atoms in the range dmin - dmax

ANGLE      dmin    dmax	Bond angles are calculated for all the indicated atoms having distances in the range dmin - dmax.

TOR	The atoms define a chain of at least four atoms. A list of torsion angles is generated for all torsion angles defined by this chain.

APP				The angle between two planes is calculated.

ARP				The angle between a line and a plane is calculated

ALL				The angle between two lines is calculated

DPA				Distances of atoms from a plane are listed.

DRA				Distances of atoms from a line are listed.

PM				Calculate the barycentre of the indicates atoms.

SAVE  				The subsequent bond length and angle calculation is stored.

CIF  	A Crystallographic Information File file 'structure_name.cif' is created containing the bond lengths and angles stored in memory in the self-defining STAR format.

TABLE   nc    ndev    nfix	The tables of bond lengths and angles previously stored are printed, ready for publication.

nc  number of columns (default=2)

ndev   number of digits for esd (default=1)

nfix     = 0  decimal digits depending on esd (default)

=  1  number (3) of decimal digits is fixed

END				end the directives input.

Examples :

#geometry

or

#geometry standard

atom                  		all the atoms are considered

dist                  			bond distances in the default range 0.2-1.7

atom                  		all the atoms

angle                 		angles between atoms in the default distance range 0.2-1.7

atom c(1)

atom c(2)

atom c(3)

atom c(4)

atom c(5)

atom c(6)

atom o(6)             		the sequence c(1), c(2),..,o(6) defines a chain

torsion               		torsion angles are computed

atom                  		all the atoms are selected

pqm  p1               		l.s. mean plane labelled 'p1' is calculated

atom c(1) until c(5)

atom o(5)             		the atoms between c(1) and c(5) plus o(5) are selected

pqm p2                		l.s. mean plane labelled 'p2'

plane p1

plane p2

app                   		the angle between the planes 'p1'^'p2' is calculated

line o(1) c(1)        		the line through the atoms o(1) and c(1) is defined

plane p2

arp                   			the angle between the line and the plane 'p2' is calculated

atom o(1)

plane p2

dpa                   		the distance of o(1) from the plane 'p2' is computed

line o(1) c(1)

atom c(7)

dra                   			the distance of c(7) from the line o(1)-c(1) is computed

atom c(1)

atom c(2)

atom o(5)

p3p  p3               		the plane 'p3' through c(1),c(2) and o(5) is computed

line o(1) c(1)

plane p3

arp                   			the angle between the line o(1)-c(1) and the plane 'p3' is calculated

line o(1) c(1)

line o(1) c(7)

all                   			the angle between lines o(1)-c(1) and o(1)-c(7) is computed

line o(1) c(1)

atom c(1)

pnr  p4               		the plane perpendicular to the line o(1)-c(1) and

passing through the atoms c(1) is computed

plane p4

plane p3

app                   		angle between planes 'p4' and 'p3' is calculated

save                  		the following distance and angle calculated are stored

atom br(1) until c(12)		the atoms between br(1) and c(12) are selected

dist 0.2   2.2        		distances in the range 0.2-2.2 are calculated

atom c(1) until h(10)

dist 0.1   1.1         		distances in the range 0.1-1.1

atom br(1) until c(12)

angle 0.2  2.2        		angles between atoms in the distance range 0.2-2.2

atom c(1) until h(10)

angle 0.1  1.1

table                 		distance and angle tables are printed

end

#end

## Molecular drawing

To plot the structure on the screen (Windows 95 platform only) or on a plotter. The command is

#PLOT

For the Windows95 platform, the default is the output on the screen. Strike any key to exit from this graphic screen. To obtain a file in HP-GL format to print on a plotter, the first instruction is:

PLOTTER

(file name)

For Unix and OpenVMS, the default is a HP-GL file, so that the first request is the (file name).

Follow the sections:

SCENA               (define the object to draw)

ORIENT              (give the plane of drawing)

If the previous sections (SCENA and ORIENT) are not given, the default condition is:

all the atoms are considered and the projection plane is the plane a^b. If SIR97 was run using the same binary access file of the structure used by CAOS, the projection plane is that defined in the windows of SIR97. Furthermore, in this conditions the following keywords STERESCOPIC and PACKING are available to obtain a stereoscopic vision of the structure and the view of the cell content respectively.

SKETCH             (simple sketch)

BALLS                (sticks and balls)

ELLIPSOIDS      (sticks and ellipsoids)

LABEL                (write the label of atoms)

HYDROGEN       (allow to draw particular bonds)

I

SCENA     0ptional



ORIENT     Optional



SKETCH   or    BALLS   or    ELLIPS



LABEL         Optional



HYDROGEN   Optional

SCENA section:

SCENA

The allowed keywords are:

ATOMS	the named atoms are considered, this instruction must precede the following keywords

TRANSFER       	transfer the indicated atoms from list 5 to a temporary list for drawing;

DELETE         	do not consider the indicated atoms;

ONLY           	consider only the indicated atoms;

MOLECULE       d(min) d(max)     	starting from a fragment of atoms (indicated using the above instructions) change the co-ordinates of the atoms present in list 5 in order to build the molecule. Two atoms are considered bonded if the distances is in the range between d(min) e d(max) .

PACKING       	apply the symmetry operators to obtain the cell content.

SOPERATION   a11 a12 ... a33 t1 t2 t3   	apply the matrix to the indicated atoms;

SCODE  NSYM  NPL  t1 t2 t3    	apply to the indicated atoms the NSYM operator (see list 2) , the NPL lattice operator (see Geometry) and the shift vector (t1 t2 t3) ;

ADDITION    n    	this keyword does not need ATOMS keyword, but n records containing co-ordinates must follow:

Ty(1) x y z b

:::::::::::::

Ty(n) x y z b

NEXT	ends the section:

ORIENT section:

ORIENT

The allowed keywords are:

IN          (atom)  	the named atoms are included

OUT      (atom) 	the named atoms are excluded

PLANE   AT(1) AT(2)  AT(3)    	chose the projection plane:

u = AT(1) - AT(2)

v = AT(1) - AT(3)

the orthogonal system is:

x parallel to u

z normal to the plane u^v

y normal to the plane x^z

PROJECTION    CODE      	CODE defines the projection plane:

CODE = XY   plane a^b

XZ   plane a^c

YZ   plane b^c ;

XROTATION       a  	apply a rotation around the x axis of an angle a ;

YROTATION       a   	as above for y axis ;

ZROTATION       a   	as above for z axis ;

VROTATION       a    AT(1) AT(2) AT(3)   	apply a rotation around the vector defined by the atoms (AT(1)-AT(2)) having the origin in the atom AT(3) ;

SKALE                k     	apply a scale factor in cm/Å;

DSKALE              k   	reduction factor (default = 0.9) .

NEXT	ends the section:

SKETCH section:

SKETCH

The default condition is: all the atoms are considered and the bonds to be drawn are defined by the connectivity present in the list 5. The allowed keywords are:

IN          (atom)  	the named atoms are included

OUT      (atom) 	the named atoms are excluded

BOND      d(min)  d(max)  Ty1  Ty2      	define the bonds to be drawn in the range d(min) and d(max): if Ty1 e Ty2 are not present, the program takes into account all the atoms considered; if Ty2 is not present, only atoms having species Ty1 are considered ; if Ty1 and Ty2 are present, (indicating only chemical species) the bonds are between atoms of Ty1 species and atoms of Ty2 species; if Ty1 e Ty2 indicate two specific atoms, only that bonds will be drawn.

NOBOND    Ty1 Ty2    	bond involving atoms of species Ty1 (and Ty2 if present) are not drawn; if Ty1 e Ty2 indicate two specific atoms, that bond will be not drawn.

EBOND       Ty1  Ty2    	draw the bond between the atoms Ty1 e Ty2 .

NEXT	ends the section:

BALLS section:

BALLS

The default condition is: all the atoms are considered and the bonds to be drawn are defined by the connectivity present in the list 5. The allowed keywords are:

IN          (atom)  	the named atoms are included

OUT      (atom) 	the named atoms are excluded

BOND       d(min)  d(max)  Ty1  Ty2      	define the bonds to be drawn in the range d(min) and d(max): if Ty1 e Ty2 are not present, the program takes into account all the atoms considered; if Ty2 is not present, only atoms having species Ty1 are considered ; if Ty1 and Ty2 are present, (indicating only chemical species) the bonds are between atoms of Ty1 species and atoms of Ty2 species; if Ty1 e Ty2 indicate two specific atoms, only that bonds will be drawn.

NOBOND          Ty1 Ty2    	bond involving atoms of species Ty1 (and Ty2 if present) are not drawn; if Ty1 e Ty2 indicate two specific atoms, that bond will be not drawn.

EBOND            Ty1  Ty2    	draw the bond between the atoms Ty1 e Ty2 .

RADIUS          r     Ty    	assign a radius r to all the atoms , if Ty is not present; only to the atoms having species Ty ; to the atom Ty if a specific atom is indicated; (default: r=3.0 for H, r=7.0 for species until Z=13, r=9.0 for the remaning species)

BISOTROPIC       Ty    	assign a radius calculated from the thermal factor B(iso) to all the atoms , if Ty is not present; only to the atoms having species Ty ; to the atom Ty if a specific atom is indicated;

LINE        ngen  rm  	draw black bonds modifying the number of lines between the atoms: ngen is in the range 1 ÷ 15 (default 6) ; rm in the range between  0.03 ÷ 0.06 mm (default 0.05) ;

NEXT	ends the section:

ELLIPSOIDS section:

ELLIPSOIDS

The default condition is: all the atoms are considered and the bonds to be drawn are defined by the connectivity present in the list 5. The allowed keywords are:

IN          (atom)  	the named atoms are included

OUT      (atom) 	the named atoms are excluded

BOND    d(min)  d(max)  Ty1  Ty2      	define the bonds to be drawn in the range d(min) and d(max): if Ty1 e Ty2 are not present, the program takes into account all the atoms considered; if Ty2 is not present, only atoms having species Ty1 are considered ; if Ty1 and Ty2 are present, (indicating only chemical species) the bonds are between atoms of Ty1 species and atoms of Ty2 species; if Ty1 e Ty2 indicate two specific atoms, only that bonds will be drawn.

NOBOND        Ty1 Ty2    	bond involving atoms of species Ty1 (and Ty2 if present) are not drawn; if Ty1 e Ty2 indicate two specific atoms, that bond will be not drawn.

EBOND          Ty1  Ty2    	draw the bond between the atoms Ty1 e Ty2 .

RADIUS          r     Ty    	assign a radius r to all the atoms , if Ty is not present; only to the atoms having species Ty ; to the atom Ty if a specific atom is indicated;

LINE        ngen  rm  	draw black bonds modifying the number of lines between the atoms: ngen is in the range 1 ÷ 15 (default 6) ; rm in the range between  0.03 ÷ 0.06 mm (default 0.05) ;

BEQUIVALENT        C   	give the coefficient defining the percentage of thermal ellipsoids (D.B.  Owen, Handbook  of  Statistical Tables, Addison-Wesley, Reading, Mass., 1962);

C = 1.5382 correspond to 50% ;

C = 1.1932 correspond to 30% .

NEXT	ends the section:

LABEL section:

##### LABEL

The default condition is: the label of all the atoms are drawn using h=0.2cm. The allowed keywords are:

CAPTION       h      iot    Ty   	h is the height (cm) of the characters, (acceptable values are in the range 0.2 ÷ 0.5 cm); iot = 0 means that the Ty chemical species only is drawn ; iot = 1 , the label of the atoms having Ty chemical species is drawn.

SYMBOL     h    CODE   Ty   	h is the symbol height (cm) to indicate the Ty atoms, (acceptable values are in the range  0.02 ÷ 0.05 cm); keyword CODE can be:

STAR

SQUARE

TRIANGLE

HEXAGON

OCTAGON

CIRCLE

NEXT	ends the section:

HYDROGEN section:

HYDROGEN

The allowed keywords are:

IN          (atom)  	the named atoms are included

HBOND       Ty1  Ty2      	define the bonds to be drawn

LINE     i      	to dash the bond lines, i is a integer number in the range 1 ÷ 6 ;

PEN     j      	j is the position number of the pen in the carousel of the HP plotter in the range 1 ÷ 6;

NEXT	ends the section:

END	end the command.

Examples:

#PLOT

BALL

END

#END

#PLOT

BALL

NEXT

LABEL

END

#END

#PLOT

ELLIPSOIDS

END

#END

#PLOT

STEREO

BALL

END

#END

#PLOT

SKETCH

END

#END

#PLOT

SCENA

ATOM  O(1) UNTIL O(4)

TRANSF

ATOM N(1)

TRANSF

ATOM C(1)  UNTIL C(37)

TRANSF

ATOM O(101)  UNTIL C(137)

TRANSF

NEXT

ORIENT

PLANE  C(6) C(3) C(7)

XROT  -65.0

XROT  -15.0

SKALE

DSKALE  0.8

NEXT

BALL

BOND 1. 1.6

RADIUS  8.  O

RADIUS  6.  N

RADIUS  4.  C

LINE  4   0.03

NEXT

LABEL

CAPTIONS  0.3  1  O

CAPTIONS  0.3  1  N

NEXT

END

#END

#PLOT

SCENA

ATOM  H

DELETE

ATOM

PACKING

ADD  4

V(1)  0. 1. 0.   1.

V(2)  1. 1. 0.   1.

V(3)  0. 1. 1.   1.

V(4)  1. 1. 1.   1.

NEXT

ORIENT

PROJECTION   XZ

ZROT  90.

SKALE

DSKALE  0.9

NEXT

SKETCH

BOND  1.1  1.6   C

BOND  1.1  1.5   C  O

BOND  1.3  1.5   C  N

BOND  1.4  1.8   N

EBOND V(1)  V(2)

EBOND V(1)  V(3)

EBOND V(4)  V(3)

EBOND V(4)  V(2)

NEXT

LABEL

CAPTION  0.2    V

SYMBOL   0.05  STAR  O

SYMBOL   0.1   TRIA  N

SYMBOL   0.01  CIRC  V

NEXT

END

#END

#PLOT

PLOTTER

(file).(ext)

SCENA

ATOM  GA(1) UNTIL S(3)

PACKING

ADD 4

V(1) 0.5 0.0 0.0 3.0

V(2) 0.0 0.5 0.0 3.0

V(3) 0.5 0.0 1.0 3.0

V(4) 0.0 0.5 1.0 3.0

NEXT

ORIENT

PLANE V(1) V(2) V(3)

XROT 0.4

ZROT 90.

SKALE 1.11111

NEXT

BALL

OUT V

RADIUS  12.0 FE

RADIUS   8.0  GA

RADIUS   24.  S

BOND   2. 3. GA S

BOND   2. 3. FE S

NEXT

LABEL

SYMB  0.01 CIRCLE V

NEXT

END

#END

#PLOT

PLOTTER

PLOT.OUT

SCENA

ATOMS   O(1) until C(41)

TRANSF

NEXT

ORIENT

PLANE N(2) C(15) O(4)

DSKALE 0.8

# XROT 0.0

XROT 45

# YROT 0.0

YROT 20

NEXT

BALL

BOND .8 1.6 C

BOND .8 1.6 C N

BOND .8 1.6 N O

BOND .8 1.6 C O

BOND .8 1.6 N

# BEQUIV 1.5382

RADIUS 5.

LINE 10  0.04

NEXT

LABEL

CAPTIONS 0.3 1 K

CAPTIONS 0.3 1 B

CAPTIONS 0.3 1 O

CAPTIONS 0.3 1 N

CAPTIONS 0.3 1 C

END

#END

#PLOT

SCENA

ATOMS   HG(1) until C(13)

TRANSF

NEXT

ORIENT

PLANE N(2) N(4) N(5)

DSKALE 0.8

XROT 26

YROT 68

NEXT

BALLS

BOND .8 1.6 C

BOND .8 1.6 C N

BOND .8 1.6 B N

BOND .8 1.9 C S

BOND .8 1.6 N

BOND .8 2.5 HG N

BOND .8 2.5 HG S

RADIUS 9. HG

RADIUS 5. N

RADIUS 6. S

RADIUS 5. C

RADIUS 5. B

LINE 10  0.04

NEXT

END

#END

## On  Windows ‘95 and Windows NT (workstation) platform

- copy the file  CAOS.ZIP in a new directory ( i.e. c:\caos )

- unzip CAOS.ZIP

- install Caos program on your DeskTop using the standard link mechanism of Windows ’95 or NT

- to run the program click on Caos icon

## On  UNIX platform

- - extract files in caos.tar, i.e.:

- - uncompress caos.tar.Z

- - tar xvf caos.tar

- copy the files SirWare.* and CaosWare.*  in your HOME directory

- create the executable file using the command

make